- Appendix No. 3 SWZ

  - DESCRIPTION OF THE SUBJECT OF THE ORDER – TECHNICAL SPECIFICATION – MINIMAL REQUIREMENTS

Delivery of a system of optical sensors allowing for a non-invasive monitoring of microorganism growth in real time

| NO. | MINIMAL REQUIRED PARAMETERS | Offered parameters (to be filled by a contractor) The contractor is requested to confirm the parameters required by the purchaser by filling in column C in the table: „yes” or „no”. The contractor is requested to specify the warranty period (pos. 15.) |
| --- | --- | --- |
| A | B | ### C |
| - Technical parameters |  |  |
|  | Measurement principle: biomass measurements using backscattered light | YES/NO |
|  | Measurements in backscattered light units can be converted to commonly used optical density at 600 nm (OD600) values | YES/NO |
|  | Measurement range: OD600 from 0.2 to 50 | YES/NO Measurement range: OD600 …………………………….. (To be filled by a contractor) |
|  | Non-invasive measurements – without the need to open cultivation vessels while taking the measurements | YES/NO |
|  | Measurements do not require sample collection | YES/NO |
|  | Measurements available for microorganism exhibiting dispersed and/or/both mycelial type of growth | YES/NO |
|  | Number of experiments: 9 or more cultivation flasks in parallel | YES/NO Number of experiments: ………………… cultivation flasks in parallel (To be filled by a contractor) |
|  | Possibility of upgrading the system in the future to allow larger number of parallel measurements | YES/NO Possibility of upgrading the system in the future to allow larger number of parallel measurements |
|  | Vessel compatibility: - common 250-ml flatbottom shake flasks (baffled and non-baffled), plastic and glass - other common cultivations flasks with a volume range: from 100 to 5000 ml | YES/NO Compatibility with the cultivation flasks: - common 250-ml flatbottom shake flasks (baffled and non-baffled), plastic and glass - other common cultivations flasks with a volume range: ………………………………… ml (To be filled by a contractor) |
|  | Shaking incubator compatibility: compatible with Infors HT Multitron and other commonly used shakers with sticky mat and clamp (spring) type mounts | YES/NO |
|  | Shaking speed: - from 160 to 250 rpm* for clamp (spring) mounts - from 100 to 200 rpm* for sticky mat mounts * rpm = revolutions per minute | YES/NO Shaking speed range: - ………………………………….. rpm* for clamp (spring) mounts - ………………………………….. rpm* for sticky mat mounts (To be filled by a contractor) |
|  | Dedicated computer software allowing analysis of measurement results | YES/NO |
|  | Operating temperature: from 10°C to 50°C | YES/NO Operating temperature range: …………………………………°C (To be filled by a contractor) |
|  | Operating humidity: from 10% to 80% | YES/NO Operating humidity range: ………………………………………………….% (To be filled by a contractor) |
|  | CE/FCC certification | YES/NO |

I confirm that the offered equipment meets all the above-mentioned parameters and requirements.

Attention!

This document must be signed with a qualified electronic signature or a trusted signature, or a personal signature by the person(s) authorized to represent the Contractor(s) jointly applying for the contract.

DESCRIPTION OF THE SUBJECT OF THE ORDER - TECHNICAL SPECIFICATION - MINIMUM REQUIREMENTS - to be submitted with the offer.